Gripper-Drone

PPO-Based Whole-Body Control

Technical Design Specification

Reinforcement Learning Training Pipeline

for a Quadrotor with U-Shaped Gripper/Landing Gear

Version 1.0  |  March 2026

Platform: Isaac Lab + SKRL + PPO

Table of Contents

1. System overview	3

2. Mathematical formulation	4

3. Control architecture	6

4. Observation space	7

5. Action space	9

6. Reward functions	10

7. Training curriculum	14

8. Domain randomization	17

9. PPO hyperparameters	18

10. Sim-to-real considerations	19

11. Known limitations and risks	20

12. Implementation checklist	21

# 1. System overview

## 1.1 Hardware specification

The Gripper-Drone is a 350mm wheelbase quadrotor (X-configuration) with a dual U-shaped plate mechanism mounted on the body underside. Each plate provides two functions: (1) landing gear when splayed outward, and (2) gripper jaw when closed inward. Total system mass is 1.08 kg without payload.

| Parameter | Value | Notes |
| --- | --- | --- |
| Wheelbase | 350 mm | Motor-to-motor diagonal |
| Total mass | 1.080 kg | Without payload |
| Body | 160 x 100 x 45 mm | Central frame |
| Propellers | 5" x 4 | r = 65 mm |
| Plate leg length | 110 mm | U-shape vertical extent |
| Plate leg spacing | 100 mm | Front-back distance |
| Pivot Y offset | 20 mm | From body center, on bottom face |
| Joint range | -5° to 50° | Negative = inward grip |
| Landing clearance | 114 mm @ 45° | Body to ground |
| Grip gap | 40 mm @ 0° | Between bars |

## 1.2 Degrees of freedom analysis

The quadrotor provides 4 independent control inputs (total thrust, roll torque, pitch torque, yaw torque). Due to underactuation, lateral motion requires body tilt, coupling position and orientation. The two gripper plates add 2 revolute DoF (plate_joint_left, plate_joint_right), giving 6 total actuated DoF.

For basic flight tasks (Stages 1-2), only the 4 quadrotor inputs are active; gripper joints are locked. For grasping tasks (Stages 3-5), all 6 inputs are used. This means the flight policy must be learned before gripper control is introduced.

## 1.3 Operational scenario

- Takeoff and ascend to search altitude

- Detect target object via forward camera (not learned; classical CV pipeline)

- Approach target using onboard depth camera for precision alignment

- Descend and grasp object with gripper plates

- Transport object to delivery location

- Release object and return to safe landing position

- Land with gripper plates in open (landing gear) configuration

Critical: This design document covers the RL training pipeline only: flight control (Stages 1-2), precision approach and grasping (Stage 3), loaded flight (Stage 4), and release (Stage 5). Object detection, mission planning, and communication are out of scope.

# 2. Mathematical formulation

## 2.1 Configuration space

The system configuration is defined as:

q = [p_b, R_wb, θ] ∈ R³ × SO(3) × T²

where:

- p_b ∈ R³: position of body center of mass in world frame (x, y, z)

- R_wb ∈ SO(3): rotation matrix from world to body frame (3x3 orthogonal, det=1)

- θ ∈ T²: two gripper plate joint angles (θ_left, θ_right)

## 2.2 Velocity space

v = [ṗ_b, Ω_b, θ̇] ∈ R⁸

- ṗ_b ∈ R³: linear velocity in world frame

- Ω_b ∈ R³: angular velocity in body frame

- θ̇ ∈ R²: joint angular velocities

## 2.3 Dynamics

The system follows coupled Euler-Lagrange dynamics:

M(q) v̇ + C(q,v) v + G(q) = τ(q, u)

- M(q): 8×8 configuration-dependent mass-inertia matrix. Changes as plates move (shifting CoM and inertia tensor).

- C(q,v)v: Coriolis and centrifugal forces. This is the primary coupling term — plate motion creates reaction torques on the body, and vice versa.

- G(q): Gravitational forces. Tilted plates create gravity torques.

- τ(q,u): Generalized forces from control inputs u = [ω_cmd, τ_θ].

The control inputs are:

u = [ω_cmd ∈ R⁴_≥0, τ_θ ∈ R²]

where ω_cmd are rotor speed commands and τ_θ are plate joint torques.

Note: The RL policy does NOT learn these dynamics explicitly. It learns a mapping from observations to high-level commands, which are then converted to u by low-level controllers. The dynamics are implicitly captured through millions of simulation interactions.

## 2.4 End-effector definition

Unlike DSAM (which has a rigid arm with a gripper tip), our system's 'end-effector' is the grip zone — the volume between the two plate bars when closed. We define the grip reference point as:

p_grip = p_b + R_wb * [0, 0, -(h_body/2 + L_leg + h_bar/2)]

where L_leg = 0.11m, h_bar = 0.012m, h_body = 0.045m. This point is fixed relative to the body (plates do not translate, only rotate). The grip zone position is controlled entirely by moving the drone body.

Critical: This is fundamentally different from DSAM. DSAM can adjust end-effector pose independently via arm joints. Our system cannot — grip position = body position + fixed offset. This simplifies the control problem (no whole-body coordination for positioning) but limits manipulation dexterity.

## 2.5 Control objective

Given a desired grip zone position p_goal(t) and optional desired body orientation R_goal(t), find control inputs that minimize:

J = ∑_t [ w_p ||p_grip(t) - p_goal(t)||  +  w_R d_geo(R_wb(t), R_goal(t)) ]

where d_geo is the geodesic distance on SO(3):

d_geo(R_1, R_2) = arccos( (tr(R_1ᵀ R_2) - 1) / 2 )

subject to actuator constraints, stability requirements, and (in grasping phases) contact force constraints.

# 3. Control architecture

## 3.1 Hierarchical structure

Following the DSAM approach (Deshmukh et al. 2025), we employ a hierarchical controller:

- RL Policy (outer loop, 150 Hz): MLP that outputs high-level commands

- Attitude controller (inner loop, 300 Hz): INDI-based, converts acceleration/attitude commands to rotor speeds

- Plate PID controller (inner loop, 300 Hz): Tracks plate joint angle targets

## 3.2 Why hierarchical

Direct motor-level control (RL outputs 4 rotor speeds + 2 joint torques) makes sim-to-real transfer extremely difficult because:

- Motor dynamics (first-order lag, saturation) vary between sim and real

- Attitude control requires high bandwidth (300+ Hz) that RL inference cannot match reliably

- INDI attitude controllers are proven robust to model mismatches in rotational dynamics

By having the RL policy output at a higher abstraction level (acceleration, attitude, joint angles), the inner-loop controllers absorb the modeling uncertainties. The RL policy focuses on the coordination problem.

## 3.3 Policy network

Multi-Layer Perceptron (MLP):

- Architecture: 512 → 256 → 128 neurons

- Activation: ELU (smooth, non-zero gradient for negative inputs)

- Output: 8 or 9 dimensions depending on stage (see Section 5)

- Inference: ONNX Runtime, target < 0.5 ms per step

Note: The network is deliberately small. For a 29-dimensional observation and 8-dimensional action, larger networks offer diminishing returns and slower inference. The DSAM paper achieved centimeter-level accuracy with this exact architecture.

# 4. Observation space

## 4.1 Stage 1-2 observations (flight only, 23 dimensions)

| Component | Dim | Description and justification |
| --- | --- | --- |
| v_b (body frame) | 3 | Linear velocity in body frame. Essential for velocity control and damping. |
| Ω_b (body frame) | 3 | Angular velocity. DSAM ablation proved this is critical for coupled systems: allows policy to detect and compensate for disturbance torques. |
| R_wb (rotation matrix) | 9 | Full rotation matrix. Avoids gimbal lock (Euler) and discontinuities (quaternion wrapping). 9 values are redundant but the NN learns the SO(3) manifold structure. |
| p_goal (body frame) | 3 | Target position expressed in body frame. Directly encodes 'where to go' relative to current pose. |
| p_error (world frame) | 3 | World-frame position error (p_goal - p_body). Provides absolute direction information. |
| Previous action | 2 | Previous acceleration XY command. Helps learn smooth transitions (action regularization). |

Total: 23 dimensions

## 4.2 Stage 3-5 observations (grasping, 31 dimensions)

Stage 1-2 observations (23) plus:

| Added component | Dim | Description |
| --- | --- | --- |
| θ_plates (normalized) | 2 | Current plate joint angles, normalized to [-1, 1]. Needed for gripper state awareness. |
| p_obj (body frame) | 3 | Object position relative to body. From depth camera + detection pipeline (simulated with noise in training). |
| grasp_flag | 1 | Binary: object is grasped (1) or not (0). Changes dynamics post-grasp. |
| payload_mass_est | 1 | Estimated payload mass (from thrust/acceleration ratio). Helps adapt to variable loads. |
| d_obj | 1 | Scalar distance to object. Explicit proximity signal for speed regulation. |

Total: 31 dimensions

## 4.3 Observation normalization

All observations are normalized using running mean and variance statistics collected during training (identical to DSAM):

o_norm = (o - μ_running) / (σ_running + ε)

where ε = 1e-5. Statistics are frozen at deployment. This is critical for sim-to-real: raw observation scales differ wildly (position in meters vs rotation matrix entries near ±1).

Critical: Do NOT clip normalized observations. Clipping destroys information about extreme states that the policy needs to handle (e.g., recovery from large disturbances). If the policy encounters out-of-distribution observations, domain randomization should be widened instead.

# 5. Action space

## 5.1 Stage 1-2 actions (flight only, 7 dimensions)

| Component | Dim | Range | Downstream controller |
| --- | --- | --- | --- |
| a_b (linear accel) | 3 | [-1, 1] → [-8, 8] m/s² | Thrust vector decomposition → desired attitude |
| Ω_cmd (body rates) | 3 | [-1, 1] → [-3, 3] rad/s | Feedforward to INDI attitude controller |
| ψ_r (yaw reference) | 1 | [-1, 1] → [-π, π] rad | Attitude controller yaw target |

## 5.2 Stage 3-5 actions (grasping, 8 dimensions)

Stage 1-2 actions (7) plus:

| Added component | Dim | Range | Downstream controller |
| --- | --- | --- | --- |
| θ_plates_cmd | 1 | [-1, 1] → [-5°, 50°] | PID joint controller (symmetric) |

Note: both plates receive the same angle command (symmetric grasp). This is a deliberate simplification: asymmetric grasping is rarely needed for the target use cases and reduces action space complexity.

## 5.3 Action scaling

The policy outputs raw actions in [-1, 1] (tanh output layer). These are linearly mapped to physical ranges:

a_physical = a_low + (a_raw + 1) / 2 * (a_high - a_low)

This mapping is applied outside the policy network, in the environment wrapper. The policy always sees and outputs normalized [-1, 1] values.

# 6. Reward functions

This is the most critical section. Every reward term must be justified, and its interaction with other terms analyzed. All rewards use the negative exponential kernel following DSAM:

r = w * exp(-α * error)

Properties: bounded in (0, w], monotonically decreasing with error, smooth gradient everywhere, α controls sensitivity.

## 6.1 Stage 1: Basic flight

Goal: Navigate to a target position and hover.

### 6.1.1 Position reward

r_pos = 4.0 * exp(-1.2 * ||p_body - p_goal||_2)

Weight 4.0 (dominant). At 1m error: r_pos = 4.0 * exp(-1.2) ≈ 1.20. At 0.1m: r_pos ≈ 3.55. At 0.01m: r_pos ≈ 3.95. The gradient is strongest in the 0.05-0.5m range, which is where precision matters most.

α = 1.2 is chosen so that the reward does not saturate to zero beyond 2-3m (exp(-1.2*3) ≈ 0.027, still nonzero). This ensures the policy receives gradient signal even when far from the goal.

### 6.1.2 Velocity penalty (near goal)

r_vel = 1.0 * exp(-2.0 * ||v_body||_2 * prox)

prox = max(1.0 - ||p_body - p_goal||_2 / 1.0, 0)

Activates only within 1m of the goal. At the goal with zero velocity: r_vel = 1.0. Moving at 1 m/s at the goal: r_vel = 1.0 * exp(-2.0) ≈ 0.14. This encourages deceleration on approach without penalizing fast transit.

The proximity factor is essential. Without it, the policy learns to approach the goal slowly from far away, which is wasteful.

### 6.1.3 Orientation reward (level flight)

r_level = 1.0 * exp(-2.0 * arccos(R_wb[2,2]))

R_wb[2,2] is the cosine of the tilt angle (dot product of body Z-axis and world Z-axis). When perfectly level: arccos(1) = 0, r_level = 1.0. At 30° tilt: r_level ≈ 0.35. This gently encourages level flight without preventing necessary tilting for translation.

Weight 1.0 (not dominant). The policy must tilt to move horizontally; over-penalizing tilt would make position control sluggish.

### 6.1.4 Action smoothness

r_smooth = 0.5 * exp(-1.0 * ||a_t - a_{t-1}||_2²)

Penalizes jerky commands. Squared L2 norm makes the penalty quadratic in action change. Weight 0.5 (low priority) to avoid over-constraining the policy.

### 6.1.5 Action magnitude

r_mag = 0.1 * exp(-0.5 * ||a_t||_2²)

Minimal weight. Only prevents unnecessarily aggressive commands. Does not significantly affect policy behavior.

### 6.1.6 Alive bonus

r_alive = 0.05 per step (constant)

Small constant reward for not crashing. Prevents the policy from learning 'crash early to avoid accumulated penalties.'

### 6.1.7 Stage 1 total reward

R_t = r_pos + r_vel + r_level + r_smooth + r_mag + r_alive

Maximum possible per step: 4.0 + 1.0 + 1.0 + 0.5 + 0.1 + 0.05 = 6.65

Expected at convergence: ~5.5-6.0 (imperfect tracking due to physics delays).

| Term | w | α | Rationale for weight |
| --- | --- | --- | --- |
| r_pos | 4.0 | 1.2 | Position is the primary objective. Must dominate to prevent reward hacking via other terms. |
| r_vel | 1.0 | 2.0 | Important for convergence quality but secondary to reaching the goal. |
| r_level | 1.0 | 2.0 | Equal to velocity. Level flight is desirable but must not prevent horizontal motion. |
| r_smooth | 0.5 | 1.0 | Low but nonzero. Required for sim-to-real but should not constrain exploration. |
| r_mag | 0.1 | 0.5 | Minimal. Only prevents pathological oscillation. |
| r_alive | 0.05 | - | Constant. Prevents early-termination exploitation. |

## 6.2 Stage 2: Precision approach

Goal: Descend vertically above a target point while maintaining horizontal alignment and level attitude.

Inherits all Stage 1 rewards, with modifications and additions:

### 6.2.1 Horizontal alignment (modified r_pos)

r_align = 4.0 * exp(-1.5 * ||p_body_xy - p_goal_xy||_2)

Replaces r_pos. Only penalizes XY error, not Z. α increased to 1.5 for tighter convergence because the approach task demands higher precision than general navigation.

### 6.2.2 Altitude control

r_alt = 2.0 * exp(-2.0 * |p_body_z - p_target_z|)

Separate altitude reward allows independent tuning. p_target_z decreases over time (descent profile).

### 6.2.3 Descent speed regulation

r_desc = 1.5 * exp(-1.5 * |v_z - v_target_z|)

v_target_z is the desired descent speed (negative, e.g., -0.1 m/s). Penalizes both too-fast and too-slow descent. Only active when drone is aligned (||p_xy_error|| < 0.1m), otherwise defaults to 0.

### 6.2.4 Level attitude (strengthened)

r_level = 2.0 * exp(-3.0 * arccos(R_wb[2,2]))

Weight doubled from Stage 1. α increased to 3.0. During precision approach, level attitude is much more important because the gripper must be horizontal to align with the object.

### 6.2.5 Horizontal speed suppression

r_slow_xy = 1.0 * exp(-3.0 * ||v_xy|| / max(d_goal_xy, 0.05))

Penalizes horizontal speed inversely proportional to distance from target XY. At 1m away: mild penalty. At 0.05m away: strong penalty for any horizontal drift.

## 6.3 Stage 3: Grasping

Goal: Descend from approach altitude, close gripper plates to grasp object.

Inherits Stage 2 rewards. Gripper action now active. Additional terms:

### 6.3.1 Gripper preparation

r_gripper_open = 0.3 * θ_normalized   (when not grasped and d_obj > 0.05m)

Encourages keeping gripper open during approach. Linear reward, not exponential, because we want a gentle bias, not a strong constraint. Deactivates when close to object (policy needs freedom to close).

### 6.3.2 Grasp success bonus

r_grasp = 10.0   (one-time, on successful grasp)

Large one-time reward. This is sparse but reachable because the approach rewards (r_align, r_alt) guide the policy to the correct pre-grasp position. The curriculum (Section 7) further ensures this is reachable.

Critical: Sparse rewards are fragile. If the policy cannot reach the grasping condition through random exploration within the first ~100M steps, learning will stall. The auto-grasp curriculum (Section 7.3) is essential.

### 6.3.3 Grasp hold reward

r_hold = 2.0 per step   (while object is grasped)

Constant reward per timestep while maintaining grasp. Without this, the policy may learn to grasp and immediately release (collecting the bonus) then re-grasp.

### 6.3.4 Drop penalty

r_drop = -10.0   (one-time, if object was grasped then dropped)

Equal magnitude to grasp bonus. Creates a strong incentive to maintain grip once established.

## 6.4 Stage 4: Loaded flight

Goal: Fly to delivery position while holding object.

Essentially Stage 1 with strengthened stability rewards:

r_pos = 4.0 * exp(-1.0 * ||p_body - p_delivery||_2)

r_stable = 2.5 * exp(-2.0 * ||Ω_b||_2)

r_level = 2.0 * exp(-3.0 * tilt_error)

r_hold = 1.0 per step

r_smooth = 1.0 * exp(-1.0 * ||a_t - a_{t-1}||_2²)

r_gentle = 0.5 * exp(-2.0 * ||a_b||_2²)

r_stable and r_gentle are new. They suppress aggressive angular motion and acceleration that could dislodge the payload. r_smooth weight is doubled from Stage 1.

## 6.5 Stage 5: Release

Essentially Stage 2 in reverse: approach delivery point, descend, open gripper, ascend. Same reward structure as Stage 2 with gripper reward inverted (encourage closing → encourage opening at the right moment). Details mirror Stage 3 with reversed grasp/release logic.

# 7. Training curriculum

## 7.1 Overview

Each stage initializes from the previous stage's trained weights. This is not optional — without curriculum learning, the compound task (approach + grasp + fly + release) is too complex for PPO to solve from scratch within reasonable compute budgets.

| Stage | Task | Init from | Est. steps | Est. time | Success metric |
| --- | --- | --- | --- | --- | --- |
| 1 | Basic flight | Scratch | 500M | ~2 hr | pos err < 5cm |
| 2 | Precision approach | Stage 1 | 500M | ~2-3 hr | XY < 3cm, level < 3° |
| 3a | Auto-grasp | Stage 2 | 300M | ~1 hr | 90% grasp rate |
| 3b | Full-range auto-grasp | Stage 3a | 500M | ~2 hr | 85% from 2m |
| 3c | Learned grasp | Stage 3b | 800M | ~3 hr | 80% autonomous |
| 4 | Loaded flight | Stage 1* | 500M | ~2 hr | pos err < 10cm |
| 5 | Release | Stage 2+3 | 500M | ~2 hr | 90% clean release |

* Stage 4 can alternatively use Stage 1 weights with mass randomization (see Section 7.5).

Total estimated training time: 14-17 hours on RTX 4090 with 4096 parallel environments.

## 7.2 Stage 1: Basic flight

### Episode structure

- Drone spawns at random position within 1m of (0, 0, 3) m

- Goal position sampled from: X,Y ∈ [-2, 2] m, Z ∈ [1, 4] m

- New goal every 6 seconds (timeout reset)

- Episode terminates if drone drops below 0.3m or exceeds 10m from origin

- Gripper plates locked at 45° (landing configuration, not actuated)

### Success criteria for advancing to Stage 2

- Mean position error < 5cm over last 10% of training

- Position reward converges (< 5% variation over 50M steps)

- No crashes in > 99% of episodes

## 7.3 Stage 3: Grasping curriculum

This is the most delicate stage. The three sub-stages:

### Stage 3a: Auto-grasp (close-range)

- Drone starts 0.2m above the object, already aligned

- When distance < 3cm AND speed < 0.1 m/s AND tilt < 5°: simulator AUTOMATICALLY closes gripper

- Policy only learns the final descent approach

- Object randomly placed at Z = 0.5-1.5m

### Stage 3b: Auto-grasp (full-range)

- Drone starts at random position 1-3m from object

- Same auto-grasp trigger as 3a

- Policy must navigate to the object AND achieve the trigger conditions

- Object position and size randomized

### Stage 3c: Learned grasp

- Auto-grasp trigger probability decreases: 100% → 50% → 0% over training

- Policy's gripper action (a_gripper) is now active

- Policy must learn WHEN to close the gripper, not just WHERE to position

- This is the hardest sub-stage; expect 800M+ steps

Critical: Do not skip the curriculum. Attempting Stage 3c from scratch will fail — the probability of randomly achieving the correct position + orientation + timing + gripper action is effectively zero for random exploration.

## 7.4 Stage 2: Precision approach

### Episode structure

- Drone spawns at random position, altitude 3-5m

- Target point at altitude 0.5-1.5m, random XY within [-2, 2] m

- Policy must navigate above target, then descend vertically

- Episode succeeds if drone holds position 0.2m above target for 5 seconds

- Gripper locked at 45°

## 7.5 Stage 4: Loaded flight

Two approaches, try A first:

### Approach A: Mass randomization only

If Stage 1 was trained with sufficient mass randomization (payload 0 - 500g), the Stage 1 policy may already handle loaded flight without retraining. Test this by evaluating Stage 1 policy with 200g, 400g, 600g payloads in simulation. If position error < 15cm for all cases, skip dedicated Stage 4 training.

### Approach B: Fine-tune

If approach A fails, fine-tune Stage 1 weights with:

- Payload mass always > 0 (sample from 50-500g)

- Stability rewards strengthened (see Section 6.4)

- Learning rate reduced to 50% of Stage 1 initial LR

# 8. Domain randomization

Domain randomization is the primary mechanism for sim-to-real transfer. Every parameter that differs between simulation and reality must be randomized during training.

## 8.1 Randomized parameters

| Parameter | Nominal | Range | Justification |
| --- | --- | --- | --- |
| Drone mass | 1.08 kg | ±10% | Battery charge variation, component tolerance |
| Payload mass | 0 g | 0-500 g | Target object mass range |
| Payload inertia | scaled | auto from mass | Scaled proportionally to mass |
| Motor constant | nominal | ±15% | Motor-to-motor variation, aging |
| Plate joint friction | nominal | [0.0, 1.5]× | DSAM showed this is critical for preventing oscillation |
| Plate joint stiffness | nominal | [0.75, 1.25]× | Servo compliance variation |
| CoM offset | 0 | ±5mm XYZ | Manufacturing and assembly tolerance |
| Wind force | 0 | N(0, 0.5N) per axis | Outdoor disturbances. Slowly varying (0.5 Hz). |
| Sensor noise (pos) | 0 | N(0, 2cm) | Motion capture / VIO noise |
| Sensor noise (vel) | 0 | N(0, 5cm/s) | EKF velocity estimation noise |
| Object detection noise | 0 | N(0, 3cm) | Depth camera + YOLO position error |
| Detection delay | 0 | 0-3 frames | Camera + inference pipeline latency |
| Object mass (grasping) | 200 g | 50-500 g | Target object variation |
| Object size | 6cm cube | 3-12cm | Must fit between gripper plates |
| Gripper surface friction | 1.2 | [0.5, 2.0] | Material variation (rubber pads, bare metal, etc.) |

## 8.2 Randomization schedule

Randomization is applied at environment reset (every episode). Parameters are sampled independently. All randomization is active from the start of each stage — there is no warmup period without randomization.

Critical: DSAM's ablation study proved that removing mass randomization caused 2-3× position error increase under load. Removing friction randomization caused joint oscillations. Do not disable randomization to 'speed up training' — the resulting policy will not transfer.

# 9. PPO hyperparameters

Starting from DSAM's configuration (which itself started from Rudin et al. 2022 'Learning to Walk in Minutes'):

| Parameter | Value | Notes |
| --- | --- | --- |
| Parallel environments | 4096 | GPU utilization sweet spot for RTX 4090 |
| Rollout length (n_steps) | 24 | 0.16s at 150Hz. Short rollouts = frequent updates. |
| Batch size | 4096 × 24 = 98304 | Total transitions per PPO update |
| Mini-batch size | 4096 | 24 mini-batches per epoch |
| PPO epochs | 5 | 5 passes over the rollout buffer |
| Learning rate | 3e-4 | Adam optimizer. Reduce for fine-tuning stages. |
| LR schedule | Constant | No decay. Manual reduction between stages if needed. |
| Discount (γ) | 0.99 | High discount for long-horizon tasks |
| GAE λ | 0.95 | Generalized Advantage Estimation |
| Clip ratio (ε) | 0.2 | Standard PPO clipping |
| Value loss coefficient | 1.0 | Equal weight to policy and value losses |
| Entropy coefficient | 0.01 | Small exploration bonus. Reduce if policy oscillates. |
| Max grad norm | 1.0 | Gradient clipping for stability |
| Episode max length | 6 seconds (900 steps) | Timeout reset. Short enough for fast iteration. |
| Environment frequency | 150 Hz | Policy inference rate |
| Simulation frequency | 300 Hz | Physics step rate (2× control rate) |

Note: These are starting values. Monitor learning curves. If position reward plateaus below 3.0 after 200M steps, increase learning rate or reduce α in position reward. If reward oscillates, reduce learning rate and increase entropy coefficient.

# 10. Sim-to-real considerations

## 10.1 Simulation fidelity requirements

- Rotor dynamics: Model first-order motor lag (τ ≈ 20-50ms). Without this, the policy learns instantaneous thrust changes that real motors cannot follow.

- Plate joint dynamics: Calibrate joint step response to match hardware. DSAM found this calibration essential before adding randomization.

- Contact physics: Isaac Sim's PhysX handles rigid body contact well, but friction coefficients and contact stiffness must be tuned to match the actual gripper material.

- Sensor delays: Model camera processing latency (30-100ms) as observation delay in training.

## 10.2 Deployment pipeline

- Export policy: Save MLP weights as ONNX format from SKRL

- Normalize stats: Export running mean/variance for observation normalization

- Onboard inference: ONNX Runtime on Jetson/RPi. Target < 1ms inference.

- Inner-loop controllers: INDI attitude controller + PID plate controller run on flight controller at 300+ Hz

- State estimation: EKF fusing IMU + position source (mocap or VIO) at 300 Hz

- Safety controller: Independent of learned policy. Handles workspace violations, low battery, loss of tracking.

## 10.3 Expected performance degradation

Based on DSAM results and general sim-to-real literature, expect:

- Position error: sim 3-5cm → real 5-10cm (2× increase)

- Orientation error: sim 3-5° → real 8-15° (2-3× increase)

- Grasp success rate: sim 95%+ → real 70-85% (depends heavily on object variety)

- Payload capacity: train with 500g max → reliably handle ~350g real (70% of trained max)

These are realistic estimates, not optimistic projections. Plan the mechanical design for these margins.

# 11. Known limitations and risks

## 11.1 Fundamental limitations

- No independent gripper positioning: Unlike DSAM, the grip zone moves only with the entire drone. This means the drone must hover perfectly above the object to grasp it — any lateral offset results in a miss. Wind gusts during the final approach phase are the highest risk.

- Symmetric grasp only: Both plates receive the same command. Objects that require asymmetric gripping (e.g., L-shaped) cannot be handled.

- Payload limits: 1.08 kg drone with ~50% thrust margin = ~500g max payload. In practice, with stability margin, ~350g reliably.

- No force feedback: The policy has no direct measurement of grip force. It relies on the binary grasp_flag and learned experience to maintain grip.

## 11.2 Training risks

- Reward hacking: The policy may find unintended ways to maximize reward. Example: oscillating near the goal to collect smoothness reward without converging. Mitigation: monitor actual position error alongside reward curves.

- Curriculum collapse: If Stage 2 is too different from Stage 1, the loaded weights may be worse than random. Mitigation: ensure reward structures overlap (Stage 2 includes all Stage 1 terms).

- Sparse reward stalling: Stage 3c (learned grasp) may stall if the policy cannot discover the grasp success condition. Mitigation: the 3-sub-stage curriculum with gradual auto-grasp removal.

- Domain randomization overshoot: Too-wide randomization makes learning impossible (the task changes too much between episodes). Start narrow, widen if sim-to-real gap is large.

## 11.3 What this design does NOT solve

- Object detection and classification (requires separate CV pipeline)

- Mission planning and multi-waypoint navigation

- Recovery from catastrophic failures (motor loss, structural damage)

- Multi-drone coordination

- Dynamic object grasping (moving targets)

# 12. Implementation checklist

## 12.1 Pre-training verification

- URDF loads in Isaac Lab without errors; all joints move correctly

- Thrust model produces expected hover thrust (~1.08 * 9.81 = 10.6 N total)

- Plate joints respond to position commands with correct PID behavior

- INDI attitude controller stabilizes hovering without RL policy

- Observation normalization produces values in roughly [-3, 3] range

- Reward function returns expected values for known states (hover at goal ≈ max reward)

- Environment resets correctly: no NaN states, no interpenetrating geometries

- 4096 environments run at > 50,000 steps/second

## 12.2 During training

- Monitor: total reward, position reward, orientation reward (separately)

- Monitor: position error histogram (not just mean — look at tail)

- Monitor: episode length (should stabilize, not decrease over time)

- Monitor: value function loss (should decrease; if it diverges, reduce LR)

- Monitor: entropy (should slowly decrease; if it collapses to 0, increase entropy coeff)

- Save checkpoints every 50M steps

- Evaluate periodically: run 100 episodes without exploration noise, measure success rate

## 12.3 Post-training validation

- Test on goal positions OUTSIDE the training distribution

- Test with payloads OUTSIDE the randomization range (e.g., 600g if trained to 500g)

- Test with sustained wind disturbance (constant 1N force)

- Test recovery from large initial state perturbation (30° tilt)

- Measure inference time on target hardware (must be < 1ms)

- Verify ONNX export produces identical outputs to PyTorch model

— End of document —